Report on the

FFDC Educational Visit

At Kannauj, U.P.

On the 14th and 15th of October, 2025

Centre for Rural Development and Technology (CRDT), IIT Indore

Report from the Industrial Training cum Exposure Visit — Fragrance & Flavour Development Centre (FFDC), Kannauj (U.P.)

Submitted by Sri Buddhodev Ghosh,

PhD Scholar, Centre for Rural Development and Technology (CRDT), IIT Indore

## Introduction

This short report summarises a two-day hands-on training at FFDC, Kannauj (Sponsored by IIT Indore), combining laboratory demonstrations, classroom briefings, and visits to traditional attar (natural perfume) distilleries. The programme covered quality testing (GC, GC-MS, HPLC), crop and raw material sourcing, traditional Deg-Bhapka distillation, market perspectives, and practical issues faced by family-run distilleries. Key takeaways include the value of pairing scientific analysis with craft knowledge, utilising seasonality-driven production cycles, and exploring business opportunities in value-added natural perfumes for both domestic and export markets.

Institutional role and learning activities

FFDC serves as a bridge between traditional perfumery and modern industrial practices, providing analytical services, training, improved planting materials, and market support to small and medium-sized fragrance enterprises. During the visit, participants received overview lectures from FFDC senior staff, observed instrumental quality testing, and discussed R&D and entrepreneurship supports available to producers. Practical sessions introduced the selection of raw materials, extraction methods, and commercialisation pathways.

Industry snapshot — Kannauj today

Kannauj remains a concentrated cluster of small, mostly family-run distilleries (the report notes over 200 operational units), famed for attars and hydrosols. Products range from seasonal floral attars (rose, jasmine) to woody bases (sandalwood, vetiver), and signature items include Mitti Attar (petrichor). These outputs feed both domestic markets and export channels (the Middle East, Europe, Southeast Asia). Modern units increasingly combine traditional Deg-Bhapka craft with controlled heating and instrumental QC to meet export standards.

Raw materials & production yields (summary)

A condensed table of commonly used crops and their practical characteristics (yields are approximate, taken from the original training document):

- Rosa damascena (Damask rose/ Desi Gulab) — flowers; very low oil yield (≈0.02–0.04%); primary source for gulab/rose attar. This is one of the low-yielding varieties among the approximately 2,000 species of roses.

- Jasmine (J. sambac / J. grandiflorum) — flowers; low yield (≈0.05–0.1%); prized for floral bases.

- Vetiver (Khus Grass) — roots; moderate yield (≈0.3–0.8%); provides cooling, woody bases.

- Sandalwood (Santalum album/ white and red Chandan) — heartwood (Not the soft outer part); higher oil %, acts as a fixative and base (often sourced from southern India or imported).

- Lemongrass — Used for many lemon fragrance-based products (Dishwasher, Detergent and Phynele, etc.) and consumables like herbal tea.

The centre also relies on some imported luxury inputs (e.g., agarwood/oud, certain fixatives) that supplement locally produced oils.

The Deg-Bhāpka distillation — concise process map

- Fresh raw material is charged into a copper Degasser (Deg in Local Language) with water; it is sealed to trap vapours.

- Vapour passes through a bamboo pipe (Chonga) to the Bhāpka, which contains a sandalwood base that absorbs the aromatic compounds.

- Slow heating (using a wood or dung fire) drives the steam transfer of volatiles; condensation in the Bhapka produces the perfumed oil/sandalwood mixture.

- Re-distillation and maturation follow; the final attar is stored in porous leather bottles (kuppi) to age and concentrate.

- In this way, they use three different methods of Distillation: 1. Hydrodistillation (For Gulab Jal, etc.), 2. Hydro-steam distillation (For Vativer etc.), 3. Steam distillation (Sandalwood oil, etc.)

Seasonal production calendar (practical summary)

Production in Kannauj follows crop phenology and weather cycles:

- Summer (Mar–Jun): Rose, vetiver harvesting; cooling oils and hydrosols produced.

- Monsoon (Jul–Sep): Mitti Attar production (distilled baked clay) peaks; essential oil extraction vastly reduced for many florals.

- Autumn (Oct–Nov): Marigold, lemongrass, kewra outputs; festival demand rises.

- Winter (Dec–Feb): Heavier, long-ageing blends (shamama, musk-type attars) are prepared and matured.

Kannauj — weather & climate (current snapshot + climate context)

Current short-range forecast (Kannauj, India): Hazy conditions with daytime highs around 26–30°C and lows around 11–16°C over the next week; air quality noted as very unhealthy on several days in the available forecast. (Source: 7-day weather forecast).

Climatological context (monthly/annual norms):

- Annual precipitation ranges from 780 to 925 mm, depending on the data source; most rainfall occurs during the southwest monsoon (July–September).

- The hot season peaks in May–June (monthly highs commonly reach the high 30s–low 40s °C). The incredible season (December–February) has meant daytime highs in the low- to mid-20s °C and nighttime minima that can approach single digits in cold snaps.

How do weather and climate affect Kannauj’s fragrance production?

- Flower oil yield and harvesting timing: Floral oil concentrations and aroma profiles are sensitive to temperature and moisture. Cooler mornings (pre-dawn harvest) during hotter months preserve volatile compounds; high daytime heat can accelerate volatilisation and cause lower effective yields if flowers are not processed promptly.

- Monsoon impacts: The bulk of rainfall in July–September both enables and constrains production. Heavy rains reduce the availability of sun-dried botanicals and complicate wood-fired heating for Deg-Bhapka operations. Still, monsoon is precisely when Mitti Attar (petrichor scents) is produced because the ‘wet earth’ aroma is seasonally authentic. Producers intentionally time the Mitti batches to coincide with the onset of the monsoon.

- Ageing and storage: High ambient humidity in monsoon months increases the chance of microbial growth in raw materials and slows leather kuppi drying; controlled storage (ventilated, low-humidity rooms) improves maturation and avoids spoilage of attars. Conversely, dry winter months favour long-term ageing of woody/resinous blends.

- Air quality & processing: Short-term haze/poor air quality (noted in the current forecast) can indicate higher particulate loads from dust or crop burning in the region. This can affect the outdoor handling of flowers and may necessitate covered handling to avoid contamination of delicate distillation charges. (Current forecast flagged “very unhealthy” air quality on several days.)

- Water availability: Annual rainfall variability — and recent regional concerns about monsoon deficits in parts of UP — make water management critical for distilleries (water for distillation, cleaning, and sanitation). Instituting rainwater harvesting and small on-site ponds improves resilience and product consistency, especially when seasonal rainfall is below average.

Practical recommendations (applied, low-cost measures)

- Harvest flowers in the early morning and immediately process or refrigerate briefly to reduce volatilisation losses.

- For the monsoon months, schedule Mitti Attar batches intentionally and keep floral distillation to a minimum unless drying and covered storage are available.

- Install simple rainwater harvesting and covered drying areas to reduce dependence on unpredictable rainfall and to improve raw material hygiene.

- Use basic air filters or covered handling lines during high-haze periods to prevent particulate contamination of flowers before distillation.

- Affiliation with modern-day low-cost technologies such as CRDT, IIT Indore — equipped with many innovations (e.g, Fragrance disinguiser for certain crops, low-cost refrigeration for small farmers, low-cost weather forecasting device, etc.).

Conclusion

The FFDC site visit reinforces that Kannauj’s perfume economy thrives at the interface of craft tradition and scientific quality control. Weather and seasonality are not peripheral — they are fundamental drivers of what can be produced, when, and at what quality. Small investments in water management, covered handling, and basic environmental controls would strengthen resilience against variable monsoons, heat extremes, and air-quality episodes, while preserving the region’s unique attar heritage.